Lexi Tub

Model: MG017

Installation Manual

Manual Rev Date: 06.2020

Table of Contents

General Information	- 2 -

Notice to Installers	- 2 -

Electrical Parameters	- 2 -

Electrical Requirements	- 2 -

Plumbing Requirements	- 2 -

Tub Diagrams	- 3 -

Tub Features	- 4 -

Tub Installation	- 5 -

Installation of Drain	- 5 -

Tub Operation	- 5 -

Troubleshooting	- 6 -

Cleaning and Maintenance	- 6 -

Additional information	- 6 -

To Expedite Part Requests:	- 6 -

Warranty Information & Registration:	- 6 -

Customer Reviews:	- 6 -

Notes:	- 7 -

# General Information

•	Note: All tubs need to be leveled in its final resting position and have all feet touching the ground.

•	It is advised to have tub onsite before preparing drain location.

•	Note: Unit comes with a 1500-watt inline heater. The heater comes on automatically when the water pump is engaged.

•	Manufacturer reserves the right to change specs or features at any time. Please check to confirm details. 1-866-783-2661

# Notice to Installers

THE FLEXIBLE DRAIN HOSE THAT IS INCLUDED WITH THIS UNIT IS FOR INSTALLATION INTO AN OPEN FLOOR DRAIN ONLY. SUBSTITUTING THE EXISTING DRAIN SHOE FOR A DIFFERENT ONE WILL NOT VOID THE WARRANTY AS LONG AS THERE IS NO DAMAGE OR EVIDENCE OF MISUSE TO THE BASE.

# Electrical Parameters

| Rated Voltage | Frequency | Motor Power |
| --- | --- | --- |
| 110V~120V | 60HZ | 0.745KW(1HP)/1.0KW 5.5-6.5 Amps |
| 110V~120V | 60HZ | 1500 Watt / 13-14 Amps Inline Heater (Pre-wired) |

# Electrical Requirements

•	1 dedicated 10-2 line for water pump (line 1, ground and neutral)

•	110-volt, 30-amp GFCI breaker

•	Includes 1500-watt inline water heater

# Plumbing Requirements

•	The unit is equipped with hot and cold, metal braided ½ inch water supply hose with ½ national pipe thread.

•	Need to install accessible hot and cold shutoffs with ½ -male national pipe thread (not included).

•	Supply hoses are to connect from the manifold on the unit to the shutoff valves.

•	All water jet features are pre plumbed with reinforced braided flexible supply hose

•	All fixtures and fittings should be checked for tightness as they may have come loose during transport. Access panel area advised.

# Tub Diagrams

# Tub Features

# Tub Installation

Water Manifold Controls: One cold water supply pipe, one hot water supply pipe.

Drainage: The diameter of the drain should be (greater than or equal to) 40mm. This drainage can be at a distance of 300-400mm from the location we show on the chart.

- Unpack the bathtub and move it into the desired position.

- Adjust the tub feet so the water will drain completely from the tub. All feet need to be touching the floor and tub needs to be level.

- Install the hand held spray wand

- Install the headrests.

- Ground the bathtub for safety.

# Installation of Drain

Floor drain must have trap installed

# Tub Operation

- Check the power supply and GFCI protection. Insert the plug into an interior power supply socket or hardwire to a junction box (Not Included).

- Turn on the cold and hot water shut off valves, and then turn on the handle of the cold and hot water faucet to mix the water to a desirable temperature.

- Operation of the faucets: an on/off valve has been set for the shower and whirlpool bath. Turn diverter to switch between water faucet and sprayer functions.

- Whirlpool bath massage: Turn valve on. Turn off the water when it reaches the water level. Water pump valve controls the whirlpool massage pump and jets.

- Drain water for about 10 minutes after installation to clear drain lines and test for water leaks.

- After using, shut off water and power supply.

# Troubleshooting

| Trouble | Reason | Solution |
| --- | --- | --- |
| No Power | - Power plug is not connected to the socket - Circuit breaker is not on - Breaker trips | - Tightly reconnect the plug with the socket. - Reset the circuit breaker. - Find and remove short circuit, reset the protection switch. |
| Water pipe leaks | - Poor seal ring - Poor joint connection - Water pipe is broken | - Replace - Connect it again - Replace |
| Tub light doesn’t work | - Bulb is damaged - Wires are cut off | - Replace (remove the nut and install a new bulb, then tighten it) - Connect the wire |

# Cleaning and Maintenance

- Clean the bathtub with ordinary liquid detergent and a soft cloth. Don’t sterilize the bathtub with cleaning products containing acetone or ammonia. Do not disinfect using agents with formic acid or formaldehyde.

- If scratched treat the surface with an acrylic rubbing polish.

- Wipe off the dirt and stains on surface with toothpaste and a soft cloth.

- Grind the light scratches or stains with 2000# sand paper, and then polish with polishing wax.

- If water is hard, it is suggested to clean the massage the device twice a year according to this following method:

  - Feed 40°C warm water into the bathtub until it reaches the lowest water level.

  - Mix 2g of liquid detergent for every liter of water, pour in and start the water force massage device and allow it to run for about 5 minutes.

  - Stop the water pump and drain out the water.

  - Fill the bathtub again with cold water and start the massage jets for about for 3 more minutes.

  - Stop the water pump, drain out the water, and clean the bathtub.

# Additional information

## To Expedite Part Requests:

Visit homewardbath.com/support

## Warranty Information & Registration:

A full description of the warranty is available on the Homeward Bath website.

To Register, visit homewardbath.com/warranty-registration

Note: Registration must be completed within 45 days of receipt to be valid.

## Customer Reviews:

Customer satisfaction is very important to us.

We would like to have your final installation picture(s) for publicity purposes.

Please send to Marketing at: marketing@hwbpro.com

# Notes:

22900 Miles Rd

Cleveland, OH 44128

Phone: 866-783-2661

Fax: 440-348-2336

www.homewardbath.com